Instructions for coregistration of micro CT and panoramic optical mapping

The goal of this code is to directly compare panoramic optical mapping (POM) results with CT-derived wall thickness. To do this, the POM geometry is first registered with the CT geometry in order to transfer POM-derived data to the epicardium of the CT volume. Next, wall thickness is extracted from the CT volume and similarly assigned to the CT epicardial voxels.

The sections below correspond to blocks of Matlab code within the script called “main_script.m”. I recommend running the code block-by-block with the “Run Section” button.

- Coregistration of CT and POM volumes

  - Initialize and load data – this just loads data from a specified folder (the variable “folder” needs to be set to the appropriate folder containing the segmented CT volume and POM results)

  - Rotate CT and POM volumes so they are roughly oriented in an anatomical position. This is performed as a pre-processing step to set the final registration up for success. The result is a Matlab data file called “tforms.mat”. If this file already exists and is satisfactory, skip to section 1.2.3 and load this file.

    - Use principal component analysis (PCA) on the volumes to roughly orient them. For each volume (the CT and the POM geometric silhouette), the code will bring up a GUI to manually adjust the orientation to match the anatomy. Adjust the X/Y/Z rotation sliders as desired and close the window to accept. (It is often necessary to consult the associated PowerPoint when the POM geometry is not obvious.)

    - Once these orientations have been finalized, save the rotation transformations in a mat-file called “tforms.mat”.

    - If the process described in section 1.2.1 has already been done (and the “tforms.mat” file exists), load and apply the saved rotation transformations. If not, skip this step.

  - Perform coregistration of preprocessed CT and POM silhouette volumes. This is done in a two-step process: first, paired control points are manually selected to anchor the coregistration (in section 1.3.1); then (if desired), this coregistration can be manually fine-tuned until it is acceptable (in section 1.3.2). The result is a Matlab data file called “final.mat”. If this file already exists, skip to section 1.3.4 and load this file.

    - Manually select control points with the “DualOrthoViewer” GUI tool. At least 3 point pairs are required, and once 3 points are selected, the result will be automatically logged to a file (in a folder called “RegistrationOutput”). Once the GUI is closed, the code will load a specified file with point pairs.

    - Manually adjust coregistration in the “AdjustCoRegVols” GUI tool, which allows adjustment of scaling, rotation, and translation in each dimension. Click “Save” to finalize choices, which will save the final transformation as a variable called “tfinal” in the Matlab workspace.

    - Once the coregistration has been finalized as described above, save the final transformation in a mat-file called “final.mat”.

    - If the processes described in section 1.3.1 and 1.3.2 have already been completed (and the “final.mat” file exists), load and apply the saved final coregistration transformation. If not, skip this step.

    - (Optional) Visualize the superimposed coregistered volumes (the original CT in gray, with the transformed POM silhouette in red).

  - Transfer POM data to CT geometry

    - Apply the final coregistration transformation to the POM silhouette and map to generate new versions in the CT frame of reference.

    - Once transformed, transfer POM map data to the CT volume (using a nearest-neighbor approach to account for imperfections in the transformation process).

    - (Optional) Visualize the POM data transferred to the CT volume.

- Extraction of wall thickness from CT volume

  - Preprocessing of CT volume – crop out unused background voxels (which speeds up all computation) and perform minimal Gaussian lowpass filtering to smooth out rough parts of the CT volume.

  - Generate image of CT volume alongside the “scale cube” (for use in Figures). This can be done using the known CT voxel dimensions.

  - Extract wall thickness

    - First, generate “filled” CT (make interior voxels “true”) that will be used to distinguish endocardium from epicardium.

    - Identify boundary voxels on the filtered CT (“surface voxels”), which correspond to both the endocardial and epicardial surfaces. Then, identify filled CT boundary voxels on the filled CT, which will not include the endocardium. Label surface voxels sufficiently far from the filled CT boundary as “endocardium” (a threshold of 5 voxels works well but can be adjusted as necessary). Label remaining surface voxels as “epicardium.”

    - For each voxel in the CT, calculate the Euclidean distance to the nearest endocardial voxel. Convert these voxel-wise distances to millimeters using the voxel dimensions.

    - (Optional) Show the CT with the distances color-coded and superimposed. The distance values on the epicardium are the wall thicknesses.